TABEL REGISTER HARDWARE & ARSITEKTUR TRUSTZONE (REGISF)

Smart Meter Listrik 3-Fasa Pascabayar (SPLN D3.006:2021 & B200) - STM32U585xx Arm Cortex-M33
Pemisahan Domain: SECURE WORLD vs NON-SECURE WORLD vs SISTEM & REGISTER INTERNAL
Format Kolom Resmi: Pin / Modul Fungsional | Register Terkait (Offset RM0456) | Alamat Fisik (Hex) | Bit / Field Konfigurasi

1. Tabel Register Hardware: SECURE WORLD (23 Pin)

| Pin / Modul Fungsional | Register Terkait (Offset RM0456) | Alamat Fisik (Hex) | Bit / Field Konfigurasi & Nilai Target |
| --- | --- | --- | --- |
| PC14 (TCXO_32K_IN) | 1. RCC_BDCR (+0xF0) 2. GPIOC_SECCFGR (+0x30) 3. GPIOC_MODER (+0x00) | 1. 0x56020CF0 (S) 2. 0x52020830 (S) 3. 0x52020800 (S) | 1. Bit 2 LSEBYP=1 (Bypass Osc), Bit 0 LSEON=1, Bit 5 LSECSSON=1 (Monitor TCXO) 2. Bit 14 SEC14 = 1 (Secure World Pad) 3. Bits [29:28] MODER14 = 11b (Analog Mode saat LSE Bypass) |
| PC15 (UNUSED_PC15) | 1. GPIOC_MODER (+0x00) 2. GPIOC_SECCFGR (+0x30) | 1. 0x52020800 (S) 2. 0x52020830 (S) | 1. Bits [31:30] MODER15 = 11b (Analog Mode / Zero Leakage) 2. Bit 15 SEC15 = 1 (Secure World Pad) |
| PB2 (RTC_CAL_OUT) | 1. RTC_CR (+0x18) 2. GPIOB_AFRL (+0x20) 3. GPIOB_SECCFGR (+0x30) 4. GTZC1_TZSC (+0x100) | 1. 0x56007818 (S) 2. 0x52020420 (S) 3. 0x52020430 (S) 4. 0x50032500 (S) | 1. Bit 29 COE=1 (Cal Output Enable), Bit 30 COSEL=0 (1 Hz) / 1 (512 Hz) 2. Bits [11:8] AFSEL2 = 0000b (AF0 RTC_OUT2) 3. Bit 2 SEC2 = 1 (Secure Pad) 4. Bit 18 RTCSEC = 1 (Secure Peripheral) |
| PC13 (TAMP_CASE_OPEN) | 1. TAMP_CR1 (+0x00) 2. TAMP_SECCFGR (+0x20) 3. GPIOC_SECCFGR (+0x30) 4. GTZC1_TZSC (+0x100) | 1. 0x56007C00 (S) 2. 0x56007C20 (S) 3. 0x52020830 (S) 4. 0x50032500 (S) | 1. Bit 0 TAMP1E = 1 (Enable Tamper 1 Active-Low / Tombol B1) 2. Bit 0 TAMP1SEC = 1 (Secure Tamper Resource) 3. Bit 13 SEC13 = 1 (Secure Pad) 4. Bit 19 TAMPSEC = 1 (Secure Peripheral) |
| PE6 (TAMP_TERM_COVER) | 1. TAMP_CR1 (+0x00) 2. TAMP_SECCFGR (+0x20) 3. GPIOE_SECCFGR (+0x30) 4. GTZC1_TZSC (+0x100) | 1. 0x56007C00 (S) 2. 0x56007C20 (S) 3. 0x52021030 (S) 4. 0x50032500 (S) | 1. Bit 2 TAMP3E = 1 (Enable Tamper 3 Active-Low) 2. Bit 2 TAMP3SEC = 1 (Secure Tamper Resource) 3. Bit 6 SEC6 = 1 (Secure Pad) 4. Bit 19 TAMPSEC = 1 (Secure Peripheral) |
| PE4 (SENSOR_MAGNET) | 1. TAMP_CR2 (+0x04) 2. TAMP_SECCFGR (+0x20) 3. GPIOE_SECCFGR (+0x30) 4. GTZC1_TZSC (+0x100) | 1. 0x56007C04 (S) 2. 0x56007C20 (S) 3. 0x52021030 (S) 4. 0x50032500 (S) | 1. Bit 6 TAMP7E = 1 (Enable Tamper 7 External Hall Sensor) 2. Bit 6 TAMP7SEC = 1 (Secure Tamper Resource) 3. Bit 4 SEC4 = 1 (Secure Pad) 4. Bit 19 TAMPSEC = 1 (Secure Peripheral) |
| PC3 (AC_FAIL_DET) | 1. GPIOC_MODER (+0x00) 2. GPIOC_SECCFGR (+0x30) 3. EXTI_EXTICR1 (+0x60) 4. EXTI_SECCFGR1 (+0x14) 5. EXTI_IMR1 (+0x80) 6. EXTI_FTSR1 (+0x04) | 1. 0x52020800 (S) 2. 0x52020830 (S) 3. 0x56022060 (S) 4. 0x56022014 (S) 5. 0x56022080 (S) 6. 0x56022004 (S) | 1. Bits [7:6] MODER3 = 00b (Input Mode) 2. Bit 3 SEC3 = 1 (Secure Pad) 3. Bits [27:24] EXTI3 = 0x02 (Port C Routing) 4. Bit 3 SEC3 = 1 (Secure Interrupt Vector) 5. Bit 3 IM3 = 1 (Interrupt Unmasked) 6. Bit 3 FT3 = 1 (Falling Edge Trigger) |
| PD11 (VBAT_SENSE) | 1. GPIOD_MODER (+0x00) 2. GPIOD_SECCFGR (+0x30) 3. ADC4_CHSELR (+0x28) 4. ADC4_CR (+0x08) 5. GTZC2_TZSC (+0x100) | 1. 0x52020C00 (S) 2. 0x52020C30 (S) 3. 0x56021028 (S) 4. 0x56021008 (S) 5. 0x56033100 (S) | 1. Bits [23:22] MODER11 = 11b (Analog Mode) 2. Bit 11 SEC11 = 1 (Secure Pad) 3. Bit 15 CHSEL15 = 1 (Select Channel 15) 4. Bit 0 ADEN = 1, Bit 2 ADSTART = 1 5. Bit 0 ADC4SEC = 1 (Secure Peripheral) |
| PC2 (VCAP_SENSE) | 1. GPIOC_MODER (+0x00) 2. GPIOC_SECCFGR (+0x30) 3. ADC4_CHSELR (+0x28) 4. GTZC2_TZSC (+0x100) | 1. 0x52020800 (S) 2. 0x52020830 (S) 3. 0x56021028 (S) 4. 0x56033100 (S) | 1. Bits [5:4] MODER2 = 11b (Analog Mode) 2. Bit 2 SEC2 = 1 (Secure Pad) 3. Bit 3 CHSEL3 = 1 (Select Channel 3) 4. Bit 0 ADC4SEC = 1 (Secure Peripheral) |
| PD12 (EXT_WDI_PULSE) | 1. GPIOD_MODER (+0x00) 2. GPIOD_BSRR (+0x18) 3. GPIOD_SECCFGR (+0x30) | 1. 0x52020C00 (S) 2. 0x52020C18 (S) 3. 0x52020C30 (S) | 1. Bits [25:24] MODER12 = 01b (Output Push-Pull) 2. Bit 12 BS12 = 1 (Set Pulse), Bit 28 BR12 = 1 (Reset Pulse) 3. Bit 12 SEC12 = 1 (Secure Pad) |
| PA4 (ADE_SS) | 1. GPIOA_MODER (+0x00) 2. GPIOA_OSPEEDR (+0x08) 3. GPIOA_BSRR (+0x18) 4. GPIOA_SECCFGR (+0x30) 5. GTZC1_TZSC (+0x100) | 1. 0x52020000 (S) 2. 0x52020008 (S) 3. 0x52020018 (S) 4. 0x52020030 (S) 5. 0x50032500 (S) | 1. Bits [9:8] MODER4 = 01b (Output Push-Pull) 2. Bits [9:8] OSPEED4 = 11b (Very High Speed) 3. Bit 4 BS4 = 1 (High/CS Non-Active), Bit 20 BR4 = 1 (Low/CS Active) 4. Bit 4 SEC4 = 1 (Secure Pad) 5. Bit 0 SPI1SEC = 1 (Secure Peripheral) |
| PA5 (ADE_SCK) | 1. GPIOA_MODER (+0x00) 2. GPIOA_AFRL (+0x20) 3. GPIOA_SECCFGR (+0x30) 4. SPI1_CR1 (+0x00) 5. SPI1_CFG2 (+0x0C) | 1. 0x52020000 (S) 2. 0x52020020 (S) 3. 0x52020030 (S) 4. 0x50013000 (S) 5. 0x5001300C (S) | 1. Bits [11:10] MODER5 = 10b (Alternate Function) 2. Bits [23:20] AFSEL5 = 0101b (AF5 SPI1_SCK) 3. Bit 5 SEC5 = 1 (Secure Pad) 4. Bit 0 SPE = 1 (SPI Enable) 5. Bit 22 MASTER = 1 (Master Mode) |
| PA6 (ADE_MISO) | 1. GPIOA_MODER (+0x00) 2. GPIOA_AFRL (+0x20) 3. GPIOA_SECCFGR (+0x30) 4. SPI1_RXDR (+0x30) | 1. 0x52020000 (S) 2. 0x52020020 (S) 3. 0x52020030 (S) 4. 0x50013030 (S) | 1. Bits [13:12] MODER6 = 10b (Alternate Function) 2. Bits [27:24] AFSEL6 = 0101b (AF5 SPI1_MISO) 3. Bit 6 SEC6 = 1 (Secure Pad) 4. Bits [31:0] RXDATA (Read 32-bit register) |
| PA7 (ADE_MOSI) | 1. GPIOA_MODER (+0x00) 2. GPIOA_AFRL (+0x20) 3. GPIOA_SECCFGR (+0x30) 4. SPI1_TXDR (+0x20) | 1. 0x52020000 (S) 2. 0x52020020 (S) 3. 0x52020030 (S) 4. 0x50013020 (S) | 1. Bits [15:14] MODER7 = 10b (Alternate Function) 2. Bits [31:28] AFSEL7 = 0101b (AF5 SPI1_MOSI) 3. Bit 7 SEC7 = 1 (Secure Pad) 4. Bits [31:0] TXDATA (Write 32-bit register) |
| PB5 (ADE_IRQ0) | 1. GPIOB_MODER (+0x00) 2. GPIOB_SECCFGR (+0x30) 3. EXTI_EXTICR2 (+0x64) 4. EXTI_SECCFGR1 (+0x14) 5. EXTI_IMR1 (+0x80) 6. EXTI_FTSR1 (+0x04) | 1. 0x52020400 (S) 2. 0x52020430 (S) 3. 0x56022064 (S) 4. 0x56022014 (S) 5. 0x56022080 (S) 6. 0x56022004 (S) | 1. Bits [11:10] MODER5 = 00b (Input Mode) 2. Bit 5 SEC5 = 1 (Secure Pad) 3. Bits [11:8] EXTI5 = 0x01 (Port B Routing) 4. Bit 5 SEC5 = 1 (Secure Interrupt Vector) 5. Bit 5 IM5 = 1 (Interrupt Unmasked) 6. Bit 5 FT5 = 1 (Falling Edge Trigger) |
| PB1 (ADE_IRQ1) | 1. GPIOB_MODER (+0x00) 2. GPIOB_SECCFGR (+0x30) 3. EXTI_EXTICR1 (+0x60) 4. EXTI_SECCFGR1 (+0x14) 5. EXTI_IMR1 (+0x80) 6. EXTI_FTSR1 (+0x04) | 1. 0x52020400 (S) 2. 0x52020430 (S) 3. 0x56022060 (S) 4. 0x56022014 (S) 5. 0x56022080 (S) 6. 0x56022004 (S) | 1. Bits [3:2] MODER1 = 00b (Input Mode) 2. Bit 1 SEC1 = 1 (Secure Pad) 3. Bits [11:8] EXTI1 = 0x01 (Port B Routing) 4. Bit 1 SEC1 = 1 (Secure Interrupt Vector) 5. Bit 1 IM1 = 1 (Interrupt Unmasked) 6. Bit 1 FT1 = 1 (Falling Edge Trigger) |
| PE11 (ADE_EVENT) | 1. GPIOE_MODER (+0x00) 2. GPIOE_SECCFGR (+0x30) 3. EXTI_EXTICR3 (+0x68) 4. EXTI_SECCFGR1 (+0x14) 5. EXTI_IMR1 (+0x80) 6. EXTI_FTSR1 (+0x04) | 1. 0x52021000 (S) 2. 0x52021030 (S) 3. 0x56022068 (S) 4. 0x56022014 (S) 5. 0x56022080 (S) 6. 0x56022004 (S) | 1. Bits [23:22] MODER11 = 00b (Input Mode) 2. Bit 11 SEC11 = 1 (Secure Pad) 3. Bits [27:24] EXTI11 = 0x04 (Port E Routing) 4. Bit 11 SEC11 = 1 (Secure Interrupt Vector) 5. Bit 11 IM11 = 1 (Interrupt Unmasked) 6. Bit 11 FT11 = 1 (Falling Edge Trigger) |
| PE7 (ADE_RESET) | 1. GPIOE_MODER (+0x00) 2. GPIOE_ODR (+0x14) 3. GPIOE_SECCFGR (+0x30) | 1. 0x52021000 (S) 2. 0x52021014 (S) 3. 0x52021030 (S) | 1. Bits [15:14] MODER7 = 01b (Output Push-Pull) 2. Bit 7 ODR7 = 1 (Run Mode) / 0 (Hardware Reset) 3. Bit 7 SEC7 = 1 (Secure Pad) |
| PE8 (ADE_CF3_ZX) | 1. GPIOE_MODER (+0x00) 2. GPIOE_SECCFGR (+0x30) 3. EXTI_EXTICR3 (+0x68) 4. EXTI_SECCFGR1 (+0x14) 5. EXTI_IMR1 (+0x80) 6. EXTI_RTSR1 (+0x00) & FTSR1 (+0x04) | 1. 0x52021000 (S) 2. 0x52021030 (S) 3. 0x56022068 (S) 4. 0x56022014 (S) 5. 0x56022080 (S) 6. 0x56022000 (S) & 0x56022004 (S) | 1. Bits [17:16] MODER8 = 00b (Input Digital) 2. Bit 8 SEC8 = 1 (Secure Pad) 3. Bits [3:0] EXTI8 = 0x04 (Port E Routing) 4. Bit 8 SEC8 = 1 (Secure Interrupt Vector) 5. Bit 8 IM8 = 1 (Interrupt Unmasked) 6. Bit 8 RT8 = 1 / FT8 = 1 (Zero-Crossing Rising & Falling Trigger) |
| PE9 (ADE_PM0) | 1. GPIOE_MODER (+0x00) 2. GPIOE_ODR (+0x14) 3. GPIOE_SECCFGR (+0x30) | 1. 0x52021000 (S) 2. 0x52021014 (S) 3. 0x52021030 (S) | 1. Bits [19:18] MODER9 = 01b (Output Push-Pull) 2. Bit 9 ODR9 = 0 (Run PSM0) / 1 (Sleep PSM3) 3. Bit 9 SEC9 = 1 (Secure Pad) |
| PE10 (ADE_PM1) | 1. GPIOE_MODER (+0x00) 2. GPIOE_ODR (+0x14) 3. GPIOE_SECCFGR (+0x30) | 1. 0x52021000 (S) 2. 0x52021014 (S) 3. 0x52021030 (S) | 1. Bits [21:20] MODER10 = 01b (Output Push-Pull) 2. Bit 10 ODR10 (Kendali mode daya ADE saat padam) 3. Bit 10 SEC10 = 1 (Secure Pad) |
| PE12 (ADE_CF1) | 1. GPIOE_MODER (+0x00) 2. GPIOE_PUPDR (+0x0C) 3. GPIOE_SECCFGR (+0x30) 4. EXTI_EXTICR4 (+0x6C) 5. EXTI_SECCFGR1 (+0x14) 6. EXTI_IMR1 (+0x80) 7. EXTI_FTSR1 (+0x04) | 1. 0x52021000 (S) 2. 0x5202100C (S) 3. 0x52021030 (S) 4. 0x5602206C (S) 5. 0x56022014 (S) 6. 0x56022080 (S) 7. 0x56022004 (S) | 1. Bits [25:24] MODER12 = 00b (Input Digital) 2. Bits [25:24] PUPDR12 = 01b (Pull-Up) 3. Bit 12 SEC12 = 1 (Secure Pad) 4. Bits [3:0] EXTI12 = 0x04 (Port E Routing) 5. Bit 12 SEC12 = 1 (Secure Interrupt Vector) 6. Bit 12 IM12 = 1 (Interrupt Unmasked) 7. Bit 12 FT12 = 1 (Falling Edge Pulse Trigger) |
| PE13 (ADE_CF2) | 1. GPIOE_MODER (+0x00) 2. GPIOE_PUPDR (+0x0C) 3. GPIOE_SECCFGR (+0x30) 4. EXTI_EXTICR4 (+0x6C) 5. EXTI_SECCFGR1 (+0x14) 6. EXTI_IMR1 (+0x80) 7. EXTI_FTSR1 (+0x04) | 1. 0x52021000 (S) 2. 0x5202100C (S) 3. 0x52021030 (S) 4. 0x5602206C (S) 5. 0x56022014 (S) 6. 0x56022080 (S) 7. 0x56022004 (S) | 1. Bits [27:26] MODER13 = 00b (Input Digital) 2. Bits [27:26] PUPDR13 = 01b (Pull-Up) 3. Bit 13 SEC13 = 1 (Secure Pad) 4. Bits [7:4] EXTI13 = 0x04 (Port E Routing) 5. Bit 13 SEC13 = 1 (Secure Interrupt Vector) 6. Bit 13 IM13 = 1 (Interrupt Unmasked) 7. Bit 13 FT13 = 1 (Falling Edge Pulse Trigger) |

2. Tabel Register Hardware: NON-SECURE WORLD (27 Pin)

| Pin / Modul Fungsional | Register Terkait (Offset RM0456) | Alamat Fisik (Hex) | Bit / Field Konfigurasi & Nilai Target |
| --- | --- | --- | --- |
| PH0 (OSC_IN) | 1. RCC_CR (+0x00) 2. GPIOH_MODER (+0x00) 3. GPIOH_SECCFGR (+0x30) | 1. 0x46020C00 (NS) 2. 0x42021C00 (NS) 3. 0x42021C30 (NS) | 1. Bit 16 HSEON = 1, Bit 17 HSERDY = 1 (HSE Active 16 MHz) 2. Bits [1:0] MODER0 = 11b (Analog Mode) 3. Bit 0 SEC0 = 0 (Non-Secure Pad) |
| PH1 (OSC_OUT) | 1. GPIOH_MODER (+0x00) 2. GPIOH_SECCFGR (+0x30) | 1. 0x42021C00 (NS) 2. 0x42021C30 (NS) | 1. Bits [3:2] MODER1 = 11b (Analog Mode) 2. Bit 1 SEC1 = 0 (Non-Secure Pad) |
| PC0 (LCD_I2C3_SCL) | 1. GPIOC_MODER (+0x00) 2. GPIOC_OTYPER (+0x04) 3. GPIOC_AFRL (+0x20) 4. GPIOC_SECCFGR (+0x30) 5. I2C3_TIMINGR (+0x10) | 1. 0x42020800 (NS) 2. 0x42020804 (NS) 3. 0x42020820 (NS) 4. 0x42020830 (NS) 5. 0x46002810 (NS) | 1. Bits [1:0] MODER0 = 10b (Alternate Function) 2. Bit 0 OT0 = 1 (Open-Drain) 3. Bits [3:0] AFSEL0 = 0100b (AF4 I2C3_SCL) 4. Bit 0 SEC0 = 0 (Non-Secure Pad) 5. 0x3042C7C7 (Standard Mode 100 kHz) |
| PC1 (LCD_I2C3_SDA) | 1. GPIOC_MODER (+0x00) 2. GPIOC_OTYPER (+0x04) 3. GPIOC_AFRL (+0x20) 4. GPIOC_SECCFGR (+0x30) 5. I2C3_CR1 (+0x00) | 1. 0x42020800 (NS) 2. 0x42020804 (NS) 3. 0x42020820 (NS) 4. 0x42020830 (NS) 5. 0x46002800 (NS) | 1. Bits [3:2] MODER1 = 10b (Alternate Function) 2. Bit 1 OT1 = 1 (Open-Drain) 3. Bits [7:4] AFSEL1 = 0100b (AF4 I2C3_SDA) 4. Bit 1 SEC1 = 0 (Non-Secure Pad) 5. Bit 0 PE = 1 (Peripheral Enable) |
| PD2 (LCD_PWR_EN) | 1. GPIOD_MODER (+0x00) 2. GPIOD_ODR (+0x14) 3. GPIOD_SECCFGR (+0x30) | 1. 0x42020C00 (NS) 2. 0x42020C14 (NS) 3. 0x42020C30 (NS) | 1. Bits [5:4] MODER2 = 01b (Output Push-Pull) 2. Bit 2 ODR2 = 1 (Power LCD ON), ODR2 = 0 (OFF) 3. Bit 2 SEC2 = 0 (Non-Secure Pad) |
| PD3 (LCD_BL_CTRL) | 1. GPIOD_MODER (+0x00) 2. GPIOD_ODR (+0x14) 3. GPIOD_SECCFGR (+0x30) | 1. 0x42020C00 (NS) 2. 0x42020C14 (NS) 3. 0x42020C30 (NS) | 1. Bits [7:6] MODER3 = 01b (Output Push-Pull) 2. Bit 3 ODR3 = 1 (Backlight ON), ODR3 = 0 (OFF) 3. Bit 3 SEC3 = 0 (Non-Secure Pad) |
| PA0 (BTN_SCROLL_UP) | 1. GPIOA_MODER (+0x00) 2. GPIOA_PUPDR (+0x0C) 3. GPIOA_SECCFGR (+0x30) 4. EXTI_EXTICR1 (+0x60) 5. EXTI_IMR1 (+0x80) 6. PWR_WUCR1 (+0x14) 7. PWR_WUCR2 (+0x18) 8. PWR_WUCR3 (+0x1C) | 1. 0x42020000 (NS) 2. 0x4202000C (NS) 3. 0x42020030 (NS) 4. 0x46022060 (NS) 5. 0x46022080 (NS) 6. 0x46020814 (NS) 7. 0x46020818 (NS) 8. 0x4602081C (NS) | 1. Bits [1:0] MODER0 = 00b (Input Mode) 2. Bits [1:0] PUPDR0 = 01b (Pull-Up) 3. Bit 0 SEC0 = 0 (Non-Secure Pad) 4. Bits [3:0] EXTI0 = 0x00 (Port A Line 0), IMR1[0]=1 5. Bit 0 WUPEN1 = 1 (WKUP1 Enable) 6. Bit 0 WUPP1 = 1 (Falling Edge Low Trigger) 7. Bits [1:0] WUSEL1 = 00b (Source 0: PA0) |
| PB6 (BTN_SCROLL_DN) | 1. GPIOB_MODER (+0x00) 2. GPIOB_PUPDR (+0x0C) 3. GPIOB_SECCFGR (+0x30) 4. EXTI_EXTICR2 (+0x64) 5. EXTI_IMR1 (+0x80) 6. PWR_WUCR1 (+0x14) 7. PWR_WUCR2 (+0x18) 8. PWR_WUCR3 (+0x1C) | 1. 0x42020400 (NS) 2. 0x4202040C (NS) 3. 0x42020430 (NS) 4. 0x46022064 (NS) 5. 0x46022080 (NS) 6. 0x46020814 (NS) 7. 0x46020818 (NS) 8. 0x4602081C (NS) | 1. Bits [13:12] MODER6 = 00b (Input Mode) 2. Bits [13:12] PUPDR6 = 01b (Pull-Up) 3. Bit 6 SEC6 = 0 (Non-Secure Pad) 4. Bits [19:16] EXTI6 = 0x01 (Port B Line 6), IMR1[6]=1 5. Bit 2 WUPEN3 = 1 (WKUP3 Enable, DS13086 Hal 129) 6. Bit 2 WUPP3 = 1 (Falling Edge Low Trigger) 7. Bits [5:4] WUSEL3 = 10b (Source 2: PB6) |
| PB7 (LED_ALM) | 1. GPIOB_MODER (+0x00) 2. GPIOB_BSRR (+0x18) 3. GPIOB_SECCFGR (+0x30) | 1. 0x42020400 (NS) 2. 0x42020418 (NS) 3. 0x42020430 (NS) | 1. Bits [15:14] MODER7 = 01b (Output Push-Pull) 2. Bit 7 BS7 = 1 (Set LED ON), Bit 23 BR7 = 1 (Reset LED OFF) 3. Bit 7 SEC7 = 0 (Non-Secure Pad) |
| PA2 (MODEM_TX) | 1. GPIOA_MODER (+0x00) 2. GPIOA_AFRL (+0x20) 3. GPIOA_SECCFGR (+0x30) 4. RCC_CCIPR1 (+0xE0) 5. LPUART1_PRESC (+0x1C) 6. LPUART1_BRR (+0x0C) 7. LPUART1_CR1 (+0x00) 8. LPUART1_TDR (+0x28) | 1. 0x42020000 (NS) 2. 0x42020020 (NS) 3. 0x42020030 (NS) 4. 0x46020CE0 (NS) 5. 0x4600241C (NS) 6. 0x4600240C (NS) 7. 0x46002400 (NS) 8. 0x46002428 (NS) | 1. Bits [5:4] MODER2 = 10b (AF Mode) 2. Bits [11:8] AFSEL2 = 1000b (AF8 LPUART1) 3. Bit 2 SEC2 = 0 (Non-Secure Pad) 4. Bits [11:10] LPUART1SEL = 10b (Pilih HSI16 16 MHz) 5. Bits [3:0] PRESCALER = 0000b (/1 divider) 6. BRR = (256×16MHz)/19200 = 0x0003_4155 7. Bit 0 UE=1, Bit 3 TE=1, Bit 29 FIFOEN=1 8. Bits [8:0] TDR (Transmit Data Register) |
| PA3 (MODEM_RX) | 1. GPIOA_MODER (+0x00) 2. GPIOA_PUPDR (+0x0C) 3. GPIOA_AFRL (+0x20) 4. GPIOA_SECCFGR (+0x30) 5. LPUART1_CR1 (+0x00) 6. LPUART1_CR3 (+0x08) 7. LPUART1_RDR (+0x24) | 1. 0x42020000 (NS) 2. 0x4202000C (NS) 3. 0x42020020 (NS) 4. 0x42020030 (NS) 5. 0x46002400 (NS) 6. 0x46002408 (NS) 7. 0x46002424 (NS) | 1. Bits [7:6] MODER3 = 10b (AF Mode) 2. Bits [7:6] PUPDR3 = 01b (Pull-Up) 3. Bits [15:12] AFSEL3 = 1000b (AF8 LPUART1) 4. Bit 3 SEC3 = 0 (Non-Secure Pad) 5. Bit 2 RE = 1 (Receiver Enable) 6. Bit 0 DMAR = 1 (DMA Rx Ring Buffer) 7. Bits [8:0] RDR (Receive Data Register) |
| PA9 (CONSOLE_TX_VCP) | 1. GPIOA_MODER (+0x00) 2. GPIOA_AFRH (+0x24) 3. GPIOA_SECCFGR (+0x30) 4. USART1_CR1 (+0x00) 5. USART1_BRR (+0x0C) 6. USART1_TDR (+0x28) | 1. 0x42020000 (NS) 2. 0x42020024 (NS) 3. 0x42020030 (NS) 4. 0x40013800 (NS) 5. 0x4001380C (NS) 6. 0x40013828 (NS) | 1. Bits [19:18] MODER9 = 10b (AF Mode) 2. Bits [7:4] AFSEL9 = 0111b (AF7 USART1_TX) 3. Bit 9 SEC9 = 0 (Non-Secure Pad) 4. Bit 0 UE = 1, Bit 3 TE = 1 5. BRR = 160MHz/115200 = 1389 = 0x056D (ST-LINK VCP) 6. Bits [8:0] TDR (Transmit Konsol printf) |
| PA10 (CONSOLE_RX_VCP) | 1. GPIOA_MODER (+0x00) 2. GPIOA_PUPDR (+0x0C) 3. GPIOA_AFRH (+0x24) 4. GPIOA_SECCFGR (+0x30) 5. USART1_CR1 (+0x00) 6. USART1_RDR (+0x24) | 1. 0x42020000 (NS) 2. 0x4202000C (NS) 3. 0x42020024 (NS) 4. 0x42020030 (NS) 5. 0x40013800 (NS) 6. 0x40013824 (NS) | 1. Bits [21:20] MODER10 = 10b (AF Mode) 2. Bits [21:20] PUPDR10 = 01b (Pull-Up) 3. Bits [11:8] AFSEL10 = 0111b (AF7 USART1_RX) 4. Bit 10 SEC10 = 0 (Non-Secure Pad) 5. Bit 2 RE = 1, Bit 5 RXNEIE = 1 6. Bits [8:0] RDR (Receive Konsol Karakter Interaktif) |
| PD4 (RS232_PWR_EN) | 1. GPIOD_MODER (+0x00) 2. GPIOD_ODR (+0x14) 3. GPIOD_SECCFGR (+0x30) | 1. 0x42020C00 (NS) 2. 0x42020C14 (NS) 3. 0x42020C30 (NS) | 1. Bits [9:8] MODER4 = 01b (Output Push-Pull) 2. Bit 4 ODR4 = 1 (Power ON) / 0 (FORCEOFF MAX3232) 3. Bit 4 SEC4 = 0 (Non-Secure Pad) |
| PD5 (MODEM_12V_EN) | 1. GPIOD_MODER (+0x00) 2. GPIOD_ODR (+0x14) 3. GPIOD_SECCFGR (+0x30) | 1. 0x42020C00 (NS) 2. 0x42020C14 (NS) 3. 0x42020C30 (NS) | 1. Bits [11:10] MODER5 = 01b (Output Push-Pull) 2. Bit 5 ODR5 = 1 (12V ON) / 0 (Putus Catu 12V) 3. Bit 5 SEC5 = 0 (Non-Secure Pad) |
| PC4 (OPTICAL_TX) | 1. GPIOC_MODER (+0x00) 2. GPIOC_AFRL (+0x20) 3. GPIOC_SECCFGR (+0x30) 4. USART3_CR1 (+0x00) 5. USART3_BRR (+0x0C) 6. USART3_TDR (+0x28) | 1. 0x42020800 (NS) 2. 0x42020820 (NS) 3. 0x42020830 (NS) 4. 0x40004800 (NS) 5. 0x4000480C (NS) 6. 0x40004828 (NS) | 1. Bits [9:8] MODER4 = 10b (AF Mode) 2. Bits [19:16] AFSEL4 = 0111b (AF7 USART3_TX) 3. Bit 4 SEC4 = 0 (Non-Secure Pad) 4. Bit 0 UE = 1, Bit 3 TE = 1 5. BRR = 160MHz/9600 = 16.667 = 0x411B 6. Bits [8:0] TDR (Transmit Data Register) |
| PC5 (OPTICAL_RX) | 1. GPIOC_MODER (+0x00) 2. GPIOC_PUPDR (+0x0C) 3. GPIOC_AFRL (+0x20) 4. GPIOC_SECCFGR (+0x30) 5. USART3_CR1 (+0x00) 6. USART3_RDR (+0x24) 7. PWR_WUCR1 (+0x14) 8. PWR_WUCR2 (+0x18) 9. PWR_WUCR3 (+0x1C) | 1. 0x42020800 (NS) 2. 0x4202080C (NS) 3. 0x42020820 (NS) 4. 0x42020830 (NS) 5. 0x40004800 (NS) 6. 0x40004824 (NS) 7. 0x46020814 (NS) 8. 0x46020818 (NS) 9. 0x4602081C (NS) | 1. Bits [11:10] MODER5 = 10b (AF Mode) 2. Bits [11:10] PUPDR5 = 01b (Pull-Up) 3. Bits [23:20] AFSEL5 = 0111b (AF7 USART3_RX) 4. Bit 5 SEC5 = 0 (Non-Secure Pad) 5. Bit 2 RE = 1 6. Bits [8:0] RDR (Receive Data Register) 7. Bit 4 WUPEN5 = 1 (WKUP5 Enable, DS13086 Hal 114) 8. Bit 4 WUPP5 = 1 (Falling Edge Trigger) 9. Bits [9:8] WUSEL5 = 00b (Source PC5) |
| PC8 (RELAY_TRIP) | 1. GPIOC_MODER (+0x00) 2. GPIOC_PUPDR (+0x0C) 3. GPIOC_BSRR (+0x18) 4. GPIOC_SECCFGR (+0x30) | 1. 0x42020800 (NS) 2. 0x4202080C (NS) 3. 0x42020818 (NS) 4. 0x42020830 (NS) | 1. Bits [17:16] MODER8 = 01b (Output Mode) 2. Bits [17:16] PUPDR8 = 10b (Pull-Down) 3. Bit 8 BS8 = 1 (Koil Buka Relai Shunt Trip Pulsa 100 ms) 4. Bit 8 SEC8 = 0 (Non-Secure Pad) |
| PC9 (RELAY_CLOSE) | 1. GPIOC_MODER (+0x00) 2. GPIOC_PUPDR (+0x0C) 3. GPIOC_BSRR (+0x18) 4. GPIOC_SECCFGR (+0x30) | 1. 0x42020800 (NS) 2. 0x4202080C (NS) 3. 0x42020818 (NS) 4. 0x42020830 (NS) | 1. Bits [19:18] MODER9 = 01b (Output Mode) 2. Bits [19:18] PUPDR9 = 10b (Pull-Down) 3. Bit 9 BS9 = 1 (Koil Tutup Relai Release Pulsa 100 ms) 4. Bit 9 SEC9 = 0 (Non-Secure Pad) |
| PC7 (RELAY_STATUS) | 1. GPIOC_MODER (+0x00) 2. GPIOC_PUPDR (+0x0C) 3. GPIOC_IDR (+0x10) 4. GPIOC_SECCFGR (+0x30) 5. EXTI_EXTICR2 (+0x64) | 1. 0x42020800 (NS) 2. 0x4202080C (NS) 3. 0x42020810 (NS) 4. 0x42020830 (NS) 5. 0x46022064 (NS) | 1. Bits [15:14] MODER7 = 00b (Input Mode) 2. Bits [15:14] PUPDR7 = 10b (Pull-Down) 3. Bit 7 IDR7 (Auxiliary Feedback Status: 1=Closed, 0=Open) 4. Bit 7 SEC7 = 0 (Non-Secure Pad) 5. Bits [27:24] EXTI7 = 0x02 (Port C Line 7) |
| PD8 (AUX_CTRL_OUT1) | 1. GPIOD_MODER (+0x00) 2. GPIOD_ODR (+0x14) 3. GPIOD_SECCFGR (+0x30) | 1. 0x42020C00 (NS) 2. 0x42020C14 (NS) 3. 0x42020C30 (NS) | 1. Bits [17:16] MODER8 = 01b (Output Push-Pull) 2. Bit 8 ODR8 (Driver Relai Bantu 1 Opto-SSR) 3. Bit 8 SEC8 = 0 (Non-Secure Pad) |
| PD10 (AUX_CTRL_IN1) | 1. GPIOD_MODER (+0x00) 2. GPIOD_PUPDR (+0x0C) 3. GPIOD_IDR (+0x10) 4. GPIOD_SECCFGR (+0x30) 5. EXTI_EXTICR3 (+0x68) 6. EXTI_IMR1 (+0x80) | 1. 0x42020C00 (NS) 2. 0x42020C0C (NS) 3. 0x42020C10 (NS) 4. 0x42020C30 (NS) 5. 0x46022068 (NS) 6. 0x46022080 (NS) | 1. Bits [21:20] MODER10 = 00b (Input Mode) 2. Bits [21:20] PUPDR10 = 01b (Pull-Up) 3. Bit 10 IDR10 (Status Input Digital 1) 4. Bit 10 SEC10 = 0 (Non-Secure Pad) 5. Bits [19:16] EXTI10 = 0x03 (Port D Line 10) 6. Bit 10 IM10 = 1 (Interrupt Unmasked) |
| PD14 (AUX_CTRL_IN2) | 1. GPIOD_MODER (+0x00) 2. GPIOD_PUPDR (+0x0C) 3. GPIOD_IDR (+0x10) 4. GPIOD_SECCFGR (+0x30) 5. EXTI_EXTICR4 (+0x6C) 6. EXTI_IMR1 (+0x80) | 1. 0x42020C00 (NS) 2. 0x42020C0C (NS) 3. 0x42020C10 (NS) 4. 0x42020C30 (NS) 5. 0x4602206C (NS) 6. 0x46022080 (NS) | 1. Bits [29:28] MODER14 = 00b (Input Mode) 2. Bits [29:28] PUPDR14 = 01b (Pull-Up) 3. Bit 14 IDR14 (Status Input Digital 2) 4. Bit 14 SEC14 = 0 (Non-Secure Pad) 5. Bits [19:16] EXTI14 = 0x03 (Port D Line 14) 6. Bit 14 IM14 = 1 (Interrupt Unmasked) |
| PB12 (FLASH_SPI2_CS) | 1. GPIOB_MODER (+0x00) 2. GPIOB_OSPEEDR (+0x08) 3. GPIOB_BSRR (+0x18) 4. GPIOB_SECCFGR (+0x30) 5. GTZC1_TZSC (+0x100) | 1. 0x42020400 (NS) 2. 0x42020408 (NS) 3. 0x42020418 (NS) 4. 0x42020430 (NS) 5. 0x40032500 (NS) | 1. Bits [25:24] MODER12 = 01b (Output Push-Pull) 2. Bits [25:24] OSPEED12 = 11b (Very High Speed) 3. Bit 12 BS12 = 1 (CS High), Bit 28 BR12 = 1 (CS Low) 4. Bit 12 SEC12 = 0 (Non-Secure Pad) 5. Bit 1 SPI2SEC = 0 (Non-Secure Peripheral) |
| PB13 (FLASH_SPI2_SCK) | 1. GPIOB_MODER (+0x00) 2. GPIOB_AFRH (+0x24) 3. GPIOB_SECCFGR (+0x30) 4. SPI2_CR1 (+0x00) 5. SPI2_CFG2 (+0x0C) | 1. 0x42020400 (NS) 2. 0x42020424 (NS) 3. 0x42020430 (NS) 4. 0x40003800 (NS) 5. 0x4000380C (NS) | 1. Bits [27:26] MODER13 = 10b (Alternate Function) 2. Bits [23:20] AFSEL13 = 0101b (AF5 SPI2_SCK) 3. Bit 13 SEC13 = 0 (Non-Secure Pad) 4. Bit 0 SPE = 1 (SPI Enable) 5. Bit 22 MASTER = 1 (Master Mode) |
| PB14 (FLASH_SPI2_MISO) | 1. GPIOB_MODER (+0x00) 2. GPIOB_AFRH (+0x24) 3. GPIOB_SECCFGR (+0x30) 4. SPI2_RXDR (+0x30) | 1. 0x42020400 (NS) 2. 0x42020424 (NS) 3. 0x42020430 (NS) 4. 0x40003830 (NS) | 1. Bits [29:28] MODER14 = 10b (Alternate Function) 2. Bits [27:24] AFSEL14 = 0101b (AF5 SPI2_MISO) 3. Bit 14 SEC14 = 0 (Non-Secure Pad) 4. Bits [31:0] RXDATA (Read Flash Data) |
| PB15 (FLASH_SPI2_MOSI) | 1. GPIOB_MODER (+0x00) 2. GPIOB_AFRH (+0x24) 3. GPIOB_SECCFGR (+0x30) 4. SPI2_TXDR (+0x20) | 1. 0x42020400 (NS) 2. 0x42020424 (NS) 3. 0x42020430 (NS) 4. 0x40003820 (NS) | 1. Bits [31:30] MODER15 = 10b (Alternate Function) 2. Bits [31:28] AFSEL15 = 0101b (AF5 SPI2_MOSI) 3. Bit 15 SEC15 = 0 (Non-Secure Pad) 4. Bits [31:0] TXDATA (Write Flash Data) |

3. Tabel Register Hardware: SISTEM & SINYAL BERSAMA (5 Pin)

| Pin / Modul Fungsional | Register Terkait (Offset RM0456) | Alamat Fisik (Hex) | Bit / Field Konfigurasi & Nilai Target |
| --- | --- | --- | --- |
| NRST (RESET_SYSTEM) | 1. RCC_CSR (+0xF4) | 1. 0x56020CF4 (S) 0x46020CF4 (NS) | 1. Bit 26 PINRSTF = 1 (NRST external pin reset flag) Bit 23 RMVF = 1 (Remove reset flags / clear status) |
| VBAT (VBAT_IN) | 1. PWR_BDCR1 (+0x20) 2. RCC_BDCR (+0xF0) | 1. 0x56020820 (S) 2. 0x56020CF0 (S) | 1. Bit 0 MONEN = 1 (Backup domain voltage monitor enable), Bit 1 VBATF 2. Bit 15 RTCEN = 1, Bit 0 LSEON = 1 |
| PA13 (SWDIO) | 1. GPIOA_AFRH (+0x24) | 1. 0x52020024 (S) | 1. Bits [23:20] AFSEL13 = 0000b (AF0 SWDIO Debug Interface) |
| PA14 (SWCLK) | 1. GPIOA_AFRH (+0x24) | 1. 0x52020024 (S) | 1. Bits [27:24] AFSEL14 = 0000b (AF0 SWCLK Debug Interface) |
| Rail DC (LED_POWER_ON) | 1. Hardwired Analog HW | 1. N/A (Analog Power Hardware) | 1. Jalur catu fasa hardware non-MCU (Disuplai langsung dari regulator power supply board) |

4. Tabel Register Internal Sistem: FIREWALL, BOOT, NVIC, DMA, KRIPTO, & WATCHDOG

| Pin / Modul Fungsional | Register Terkait (Offset RM0456) | Alamat Fisik (Hex) | Bit / Field Konfigurasi & Nilai Target |
| --- | --- | --- | --- |
| FLASH_OPTR (Option Register) | 1. FLASH_OPTR (+0x40) | 1. 0x40022040 (NS) 0x50022040 (S) | 1. Bit 31 TZEN = 1 (Enable TrustZone Perangkat Keras) Bits [7:0] RDP = 0xCC (Readout Protection Level 2 / JTAG Permanently Locked) Bits [10:9] BOR_LEV = 00b (Brownout Reset Level 0: 1,71 V Threshold) |
| FLASH_SECBOOTADD0R (Boot Address) | 1. FLASH_SECBOOTADD0R (+0x90) | 1. 0x50022090 (S) | 1. Bits [31:7] SECBOOTADD0 = 0x0C000000 >> 7 (Vektor Bootloader Secure Flash Bank 1) |
| FLASH_SECWM1R1..R2 (Flash Bank 1 Watermark) | 1. FLASH_SECWM1R1 (+0x80) 2. FLASH_SECWM1R2 (+0x84) | 1. 0x50022080 (S) 2. 0x50022084 (S) | 1. Bits [6:0] SECWM1_PSTRT = 0 (Page Mulai: 0) 2. Bits [22:16] SECWM1_PEND = 63 (Page Akhir: 63, Total 128 KB Flash Secure) |
| GTZC1_MPCBB1 (Firewall SRAM1 192KB) | 1. GTZC1_MPCBB1_CR (+0x00) 2. GTZC1_MPCBB1_SECCFGR0..31 (+0x100) | 1. 0x50032800 (S) 2. 0x50032900 s.d. 0x5003297C (S) | 1. Bit 0 SRWILADIS = 1 (Memicu Security Fault saat akses ilegal) 2. Blok 0..31 = 1 (Secure: Buffer ADE9000 & Kunci AES) Blok 32..383 = 0 (Non-Secure: Heap FreeRTOS & Stack DLMS) |
| GTZC1_MPCBB3 (Firewall SRAM3 512KB) | 1. GTZC1_MPCBB3_CR (+0x00) 2. GTZC1_MPCBB3_SECCFGR0..31 (+0x300) | 1. 0x50032800 (S) 2. 0x50032B00 s.d. 0x50032B7C (S) | 1. SRWILADIS = 1, INVSECSTATE = 0 2. Seluruh array SECCFGR = 0x00000000 (Seluruh 512 KB dibuka Non-Secure untuk Heap FreeRTOS & Load Profile) |
| NVIC->ITNS (Interrupt Targeting) | 1. NVIC->ITNS[0..15] | 1. 0xE000E380 s.d. 0xE000E3BC (S) | 1. ITNS bit = 1 untuk 11 Vektor Non-Secure: LPUART1, USART1, USART3, I2C1, I2C3, GPDMA1_Ch0, PVD, EXTI0, EXTI6, RTC, TAMP ITNS bit = 0 untuk Vektor Secure: EXTI3 (ADE IRQ), EXTI1, EXTI5, EXTI11, TAMP_SEC |
| SCB_NS->VTOR (Vektor Non-Secure) | 1. SCB_NS->VTOR | 1. 0xE000ED08 (NS) | 1. VTOR = 0x08100000 (Alamat Vektor Tabel Non-Secure Flash Bank 2 sebelum Secure World melompat via funcptr_NS NonSecure_ResetHandler) |
| SAU Controller (NSC Veneer Gateway) | 1. SAU->CTRL 2. SAU->RBAR / RLAR | 1. 0xE000EDD0 (S) 2. 0xE000EDD8 s.d. 0xE000EDDC (S) | 1. Bit 0 ENABLE = 1, Bit 1 ALLNS = 0 2. Region 0: Start = 0x0C0FE000, Limit = 0x0C0FFFFF (Area NSC 32 KB untuk instruksi Secure Gateway SG) |
| GPDMA1 Controller (DMA Security Channels) | 1. GPDMA1_SECCFGR (+0x00) 2. GPDMA1_PRIVCFGR (+0x04) | 1. 0x50020000 (S) 2. 0x50020004 (S) | 1. SEC0 = 0 (Channel 0 Non-Secure: RX Modem Buffer) SEC1 = 1 (Channel 1 Secure: RX SPI1 ADE9000 Metering) 2. PRIV0 = 0 (Unprivileged task access) |
| TAMP Backup & Auto-Erase (Brankas Sabotase Fisik) | 1. TAMP_BKP0R..BKP31R (+0x30) 2. TAMP_ERCFGR (+0x24) | 1. 0x56007C30 s.d. 0x56007CAC (S) 2. 0x56007C24 (S) | 1. 32 Register 32-bit (128 Byte): Kunci Master DLMS GUEK & Akumulasi kWh Terakhir di domain baterai VSW 2. Bit 0 ERCFG0 = 1 (Auto-Erase seluruh kunci & BKPxR dalam 1 siklus clock saat PC13 case open terpicu) |
| SAES Hardware Engine (DLMS/COSEM Accelerator) | 1. SAES_CR (+0x00) 2. SAES_KEYR0..3 (+0x04) | 1. 0x50060400 (S) 2. 0x50060404 s.d. 0x50060410 (S) | 1. Bit 0 EN = 1, MODE: 00b (Enkripsi) / 01b (Dekripsi), CHMOD = 011b (Mode GCM Galois/Counter) 2. 128-bit Key Register dimuat langsung dari hardware secret key |
| RNG Hardware Engine (True Random Generator) | 1. RNG_CR (+0x00) 2. RNG_DR (+0x08) | 1. 0x50060800 (S) 2. 0x50060808 (S) | 1. Bit 2 RNGEN = 1 (TRNG Enable) 2. Bits [31:0] RNDATA (Nilai acak entropi fisik termal silikon untuk Challenge DLMS Authentication) |
| PWR Low-Power & PVD (Manajemen Daya Pemadaman) | 1. PWR_CR1 (+0x00) 2. PWR_SVMCR (+0x10) 3. PWR_BDCR2 (+0x24) | 1. 0x46020800 (NS) 2. 0x46020810 (NS) 3. 0x56020824 (S) | 1. Bits [2:0] LPMS = 010b (Stop 2 Mode ~2,5 uA), Bit 8 DBP = 1 (Buka akses tulis domain RTC) 2. Bit 24 PVDE = 1, Bits [23:21] PLS = 011b (PVD Level 3: 2,5 V Last-Gasp Trigger) 3. Bit 0 VBE = 1 (Charger Superkapasitor / Baterai Cadangan ON) |
| Dual-Watchdog Supervisor (IWDG & WWDG Anti-Hang) | 1. IWDG_KR (+0x00) 2. IWDG_PR (+0x04) 3. IWDG_RLR (+0x08) 4. WWDG_CR (+0x00) | 1. 0x40003000 (NS) 2. 0x40003004 (NS) 3. 0x40003008 (NS) 4. 0x40002C00 (NS) | 1. KR = 0x5555 (Unlock), 0xCCCC (Start), 0xAAAA (Refresh) 2. PR = 100b (/64), RLR = 1000 (Timeout ~2.000 ms pada LSI 32 kHz independen) 4. Bit 7 WDGA = 1 (Window Watchdog untuk timing pengawasan task metrologi) |
| ICACHE Control (Instruction Cache 160MHz) | 1. ICACHE_CR (+0x00) | 1. 0x50030400 (S) | 1. Bit 0 EN = 1 (ICACHE Aktif penuh untuk eksekusi kode dari Flash pada 160 MHz tanpa wait-state penalty) |